/////////////////////////////////////////////////////////////////////

////                                                             ////

////  WISHBONE rev.B2 compliant I2C Master bit-controller        ////

////                                                             ////

////                                                             ////

////  Author: Richard Herveille                                  ////

////          richard@asics.ws                                   ////

////          www.asics.ws                                       ////

////                                                             ////

////  Downloaded from: http://www.opencores.org/projects/i2c/    ////

////                                                             ////

/////////////////////////////////////////////////////////////////////

////                                                             ////

//// Copyright (C) 2001 Richard Herveille                        ////

////                    richard@asics.ws                         ////

////                                                             ////

//// This source file may be used and distributed without        ////

//// restriction provided that this copyright statement is not   ////

//// removed from the file and that any derivative work contains ////

//// the original copyright notice and the associated disclaimer.////

////                                                             ////

////     THIS SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY     ////

//// EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED   ////

//// TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS   ////

//// FOR A PARTICULAR PURPOSE. IN NO EVENT SHALL THE AUTHOR      ////

//// OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT,         ////

//// INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES    ////

//// (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF SUBSTITUTE   ////

//// GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR        ////

//// BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF  ////

//// LIABILITY, WHETHER IN  CONTRACT, STRICT LIABILITY, OR TORT  ////

//// (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT  ////

//// OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE         ////

//// POSSIBILITY OF SUCH DAMAGE.                                 ////

////                                                             ////

/////////////////////////////////////////////////////////////////////

//  CVS Log

//

//  $Id: i2c_master_bit_ctrl.v,v 1.14 2009-01-20 10:25:29 rherveille Exp $

//

//  $Date: 2009-01-20 10:25:29 $

//  $Revision: 1.14 $

//  $Author: rherveille $

//  $Locker:  $

//  $State: Exp $

//

// Change History:

//               $Log: $

//               Revision 1.14  2009/01/20 10:25:29  rherveille

//               Added clock synchronization logic

//               Fixed slave_wait signal

//

//               Revision 1.13  2009/01/19 20:29:26  rherveille

//               Fixed synopsys miss spell (synopsis)

//               Fixed cr[0] register width

//               Fixed ! usage instead of ~

//               Fixed bit controller parameter width to 18bits

//

//               Revision 1.12  2006/09/04 09:08:13  rherveille

//               fixed short scl high pulse after clock stretch

//               fixed slave model not returning correct '(n)ack' signal

//

//               Revision 1.11  2004/05/07 11:02:26  rherveille

//               Fixed a bug where the core would signal an arbitration lost (AL bit set), when another master controls the bus and the other master generates a STOP bit.

//

//               Revision 1.10  2003/08/09 07:01:33  rherveille

//               Fixed a bug in the Arbitration Lost generation caused by delay on the (external) sda line.

//               Fixed a potential bug in the byte controller's host-acknowledge generation.

//

//               Revision 1.9  2003/03/10 14:26:37  rherveille

//               Fixed cmd_ack generation item (no bug).

//

//               Revision 1.8  2003/02/05 00:06:10  rherveille

//               Fixed a bug where the core would trigger an erroneous 'arbitration lost' interrupt after being reset, when the reset pulse width < 3 clk cycles.

//

//               Revision 1.7  2002/12/26 16:05:12  rherveille

//               Small code simplifications

//

//               Revision 1.6  2002/12/26 15:02:32  rherveille

//               Core is now a Multimaster I2C controller

//

//               Revision 1.5  2002/11/30 22:24:40  rherveille

//               Cleaned up code

//

//               Revision 1.4  2002/10/30 18:10:07  rherveille

//               Fixed some reported minor start/stop generation timing issuess.

//

//               Revision 1.3  2002/06/15 07:37:03  rherveille

//               Fixed a small timing bug in the bit controller.\nAdded verilog simulation environment.

//

//               Revision 1.2  2001/11/05 11:59:25  rherveille

//               Fixed wb_ack_o generation bug.

//               Fixed bug in the byte_controller statemachine.

//               Added headers.

//

//

/////////////////////////////////////

// Bit controller section

/////////////////////////////////////

//

// Translate simple commands into SCL/SDA transitions

// Each command has 5 states, A/B/C/D/idle

//

// start:	SCL	~~~~~~~~~~\____

//	SDA	~~~~~~~~\______

//		 x | A | B | C | D | i

//

// repstart	SCL	____/~~~~\___

//	SDA	__/~~~\______

//		 x | A | B | C | D | i

//

// stop	SCL	____/~~~~~~~~

//	SDA	==\____/~~~~~

//		 x | A | B | C | D | i

//

//- write	SCL	____/~~~~\____

//	SDA	==X=========X=

//		 x | A | B | C | D | i

//

//- read	SCL	____/~~~~\____

//	SDA	XXXX=====XXXX

//		 x | A | B | C | D | i

//

// Timing:     Normal mode      Fast mode

///////////////////////////////////////////////////////////////////////

// Fscl        100KHz           400KHz

// Th_scl      4.0us            0.6us   High period of SCL

// Tl_scl      4.7us            1.3us   Low period of SCL

// Tsu:sta     4.7us            0.6us   setup time for a repeated start condition

// Tsu:sto     4.0us            0.6us   setup time for a stop conditon

// Tbuf        4.7us            1.3us   Bus free time between a stop and start condition

//

// synopsys translate_off

`include "timescale.v"

// synopsys translate_on

`include "i2c_master_defines.v"

module i2c_master_bit_ctrl (

input             clk,      // system clock

input             rst,      // synchronous active high reset

input             nReset,   // asynchronous active low reset

input             ena,      // core enable signal

input      [15:0] clk_cnt,  // clock prescale value

input      [ 3:0] cmd,      // command (from byte controller)

output reg        cmd_ack,  // command complete acknowledge

output reg        busy,     // i2c bus busy

output reg        al,       // i2c bus arbitration lost

input             din,

output reg        dout,

input             scl_i,    // i2c clock line input

output            scl_o,    // i2c clock line output

output reg        scl_oen,  // i2c clock line output enable (active low)

input             sda_i,    // i2c data line input

output            sda_o,    // i2c data line output

output reg        sda_oen,  // i2c data line output enable (active low)

input	      VDD, // Power

input	      VSS // Ground

);

//

// variable declarations

//

reg [ 1:0] cSCL, cSDA;      // capture SCL and SDA

reg [ 2:0] fSCL, fSDA;      // SCL and SDA filter inputs

reg        sSCL, sSDA;      // filtered and synchronized SCL and SDA inputs

reg        dSCL, dSDA;      // delayed versions of sSCL and sSDA

reg        dscl_oen;        // delayed scl_oen

reg        sda_chk;         // check SDA output (Multi-master arbitration)

reg        clk_en;          // clock generation signals

reg        slave_wait;      // slave inserts wait states

reg [15:0] cnt;             // clock divider counter (synthesis)

reg [13:0] filter_cnt;      // clock divider for filter

// state machine variable

reg [17:0] c_state;

//

// module body

//

// whenever the slave is not ready it can delay the cycle by pulling SCL low

// delay scl_oen

always @(posedge clk)

dscl_oen <= #1 scl_oen;

// slave_wait is asserted when master wants to drive SCL high, but the slave pulls it low

// slave_wait remains asserted until the slave releases SCL

always @(posedge clk or negedge nReset)

if (!nReset) slave_wait <= 1'b0;

else         slave_wait <= (scl_oen & ~dscl_oen & ~sSCL) | (slave_wait & ~sSCL);

// master drives SCL high, but another master pulls it low

// master start counting down its low cycle now (clock synchronization)

wire scl_sync   = dSCL & ~sSCL & scl_oen;

// generate clk enable signal

always @(posedge clk or negedge nReset)

if (~nReset)

begin

cnt    <= #1 16'h0;

clk_en <= #1 1'b1;

end

else if (rst || ~|cnt || !ena || scl_sync)

begin

cnt    <= #1 clk_cnt;

clk_en <= #1 1'b1;

end

else if (slave_wait)

begin

cnt    <= #1 cnt;

clk_en <= #1 1'b0;

end

else

begin

cnt    <= #1 cnt - 16'h1;

clk_en <= #1 1'b0;

end

// generate bus status controller

// capture SDA and SCL

// reduce metastability risk

always @(posedge clk or negedge nReset)

if (!nReset)

begin

cSCL <= #1 2'b00;

cSDA <= #1 2'b00;

end

else if (rst)

begin

cSCL <= #1 2'b00;

cSDA <= #1 2'b00;

end

else

begin

cSCL <= {cSCL[0],scl_i};

cSDA <= {cSDA[0],sda_i};

end

// filter SCL and SDA signals; (attempt to) remove glitches

always @(posedge clk or negedge nReset)

if      (!nReset     ) filter_cnt <= 14'h0;

else if (rst || !ena ) filter_cnt <= 14'h0;

else if (~|filter_cnt) filter_cnt <= clk_cnt >> 2; //16x I2C bus frequency

else                   filter_cnt <= filter_cnt -1;

always @(posedge clk or negedge nReset)

if (!nReset)

begin

fSCL <= 3'b111;

fSDA <= 3'b111;

end

else if (rst)

begin

fSCL <= 3'b111;

fSDA <= 3'b111;

end

else if (~|filter_cnt)

begin

fSCL <= {fSCL[1:0],cSCL[1]};

fSDA <= {fSDA[1:0],cSDA[1]};

end

// generate filtered SCL and SDA signals

always @(posedge clk or negedge nReset)

if (~nReset)

begin

sSCL <= #1 1'b1;

sSDA <= #1 1'b1;

dSCL <= #1 1'b1;

dSDA <= #1 1'b1;

end

else if (rst)

begin

sSCL <= #1 1'b1;

sSDA <= #1 1'b1;

dSCL <= #1 1'b1;

dSDA <= #1 1'b1;

end

else

begin

sSCL <= #1 &fSCL[2:1] | &fSCL[1:0] | (fSCL[2] & fSCL[0]);

sSDA <= #1 &fSDA[2:1] | &fSDA[1:0] | (fSDA[2] & fSDA[0]);

dSCL <= #1 sSCL;

dSDA <= #1 sSDA;

end

// detect start condition => detect falling edge on SDA while SCL is high

// detect stop condition => detect rising edge on SDA while SCL is high

reg sta_condition;

reg sto_condition;

always @(posedge clk or negedge nReset)

if (~nReset)

begin

sta_condition <= #1 1'b0;

sto_condition <= #1 1'b0;

end

else if (rst)

begin

sta_condition <= #1 1'b0;

sto_condition <= #1 1'b0;

end

else

begin

sta_condition <= #1 ~sSDA &  dSDA & sSCL;

sto_condition <= #1  sSDA & ~dSDA & sSCL;

end

// generate i2c bus busy signal

always @(posedge clk or negedge nReset)

if      (!nReset) busy <= #1 1'b0;

else if (rst    ) busy <= #1 1'b0;

else              busy <= #1 (sta_condition | busy) & ~sto_condition;

// generate arbitration lost signal

// aribitration lost when:

// 1) master drives SDA high, but the i2c bus is low

// 2) stop detected while not requested

reg cmd_stop;

always @(posedge clk or negedge nReset)

if (~nReset)

cmd_stop <= #1 1'b0;

else if (rst)

cmd_stop <= #1 1'b0;

else if (clk_en)

cmd_stop <= #1 cmd == `I2C_CMD_STOP;

always @(posedge clk or negedge nReset)

if (~nReset)

al <= #1 1'b0;

else if (rst)

al <= #1 1'b0;

else

al <= #1 (sda_chk & ~sSDA & sda_oen) | (|c_state & sto_condition & ~cmd_stop);

// generate dout signal (store SDA on rising edge of SCL)

always @(posedge clk)

if (sSCL & ~dSCL) dout <= #1 sSDA;

// generate statemachine

// nxt_state decoder

parameter [17:0] idle    = 18'b0_0000_0000_0000_0000;

parameter [17:0] start_a = 18'b0_0000_0000_0000_0001;

parameter [17:0] start_b = 18'b0_0000_0000_0000_0010;

parameter [17:0] start_c = 18'b0_0000_0000_0000_0100;

parameter [17:0] start_d = 18'b0_0000_0000_0000_1000;

parameter [17:0] start_e = 18'b0_0000_0000_0001_0000;

parameter [17:0] stop_a  = 18'b0_0000_0000_0010_0000;

parameter [17:0] stop_b  = 18'b0_0000_0000_0100_0000;

parameter [17:0] stop_c  = 18'b0_0000_0000_1000_0000;

parameter [17:0] stop_d  = 18'b0_0000_0001_0000_0000;

parameter [17:0] rd_a    = 18'b0_0000_0010_0000_0000;

parameter [17:0] rd_b    = 18'b0_0000_0100_0000_0000;

parameter [17:0] rd_c    = 18'b0_0000_1000_0000_0000;

parameter [17:0] rd_d    = 18'b0_0001_0000_0000_0000;

parameter [17:0] wr_a    = 18'b0_0010_0000_0000_0000;

parameter [17:0] wr_b    = 18'b0_0100_0000_0000_0000;

parameter [17:0] wr_c    = 18'b0_1000_0000_0000_0000;

parameter [17:0] wr_d    = 18'b1_0000_0000_0000_0000;

always @(posedge clk or negedge nReset)

if (!nReset)

begin

c_state <= #1 idle;

cmd_ack <= #1 1'b0;

scl_oen <= #1 1'b1;

sda_oen <= #1 1'b1;

sda_chk <= #1 1'b0;

end

else if (rst | al)

begin

c_state <= #1 idle;

cmd_ack <= #1 1'b0;

scl_oen <= #1 1'b1;

sda_oen <= #1 1'b1;

sda_chk <= #1 1'b0;

end

else

begin

cmd_ack   <= #1 1'b0; // default no command acknowledge + assert cmd_ack only 1clk cycle

if (clk_en)

case (c_state) // synopsys full_case parallel_case

// idle state

idle:

begin

case (cmd) // synopsys full_case parallel_case

`I2C_CMD_START: c_state <= #1 start_a;

`I2C_CMD_STOP:  c_state <= #1 stop_a;

`I2C_CMD_WRITE: c_state <= #1 wr_a;

`I2C_CMD_READ:  c_state <= #1 rd_a;

default:        c_state <= #1 idle;

endcase

scl_oen <= #1 scl_oen; // keep SCL in same state

sda_oen <= #1 sda_oen; // keep SDA in same state

sda_chk <= #1 1'b0;    // don't check SDA output

end

// start

start_a:

begin

c_state <= #1 start_b;

scl_oen <= #1 scl_oen; // keep SCL in same state

sda_oen <= #1 1'b1;    // set SDA high

sda_chk <= #1 1'b0;    // don't check SDA output

end

start_b:

begin

c_state <= #1 start_c;

scl_oen <= #1 1'b1; // set SCL high

sda_oen <= #1 1'b1; // keep SDA high

sda_chk <= #1 1'b0; // don't check SDA output

end

start_c:

begin

c_state <= #1 start_d;

scl_oen <= #1 1'b1; // keep SCL high

sda_oen <= #1 1'b0; // set SDA low

sda_chk <= #1 1'b0; // don't check SDA output

end

start_d:

begin

c_state <= #1 start_e;

scl_oen <= #1 1'b1; // keep SCL high

sda_oen <= #1 1'b0; // keep SDA low

sda_chk <= #1 1'b0; // don't check SDA output

end

start_e:

begin

c_state <= #1 idle;

cmd_ack <= #1 1'b1;

scl_oen <= #1 1'b0; // set SCL low

sda_oen <= #1 1'b0; // keep SDA low

sda_chk <= #1 1'b0; // don't check SDA output

end

// stop

stop_a:

begin

c_state <= #1 stop_b;

scl_oen <= #1 1'b0; // keep SCL low

sda_oen <= #1 1'b0; // set SDA low

sda_chk <= #1 1'b0; // don't check SDA output

end

stop_b:

begin

c_state <= #1 stop_c;

scl_oen <= #1 1'b1; // set SCL high

sda_oen <= #1 1'b0; // keep SDA low

sda_chk <= #1 1'b0; // don't check SDA output

end

stop_c:

begin

c_state <= #1 stop_d;

scl_oen <= #1 1'b1; // keep SCL high

sda_oen <= #1 1'b0; // keep SDA low

sda_chk <= #1 1'b0; // don't check SDA output

end

stop_d:

begin

c_state <= #1 idle;

cmd_ack <= #1 1'b1;

scl_oen <= #1 1'b1; // keep SCL high

sda_oen <= #1 1'b1; // set SDA high

sda_chk <= #1 1'b0; // don't check SDA output

end

// read

rd_a:

begin

c_state <= #1 rd_b;

scl_oen <= #1 1'b0; // keep SCL low

sda_oen <= #1 1'b1; // tri-state SDA

sda_chk <= #1 1'b0; // don't check SDA output

end

rd_b:

begin

c_state <= #1 rd_c;

scl_oen <= #1 1'b1; // set SCL high

sda_oen <= #1 1'b1; // keep SDA tri-stated

sda_chk <= #1 1'b0; // don't check SDA output

end

rd_c:

begin

c_state <= #1 rd_d;

scl_oen <= #1 1'b1; // keep SCL high

sda_oen <= #1 1'b1; // keep SDA tri-stated

sda_chk <= #1 1'b0; // don't check SDA output

end

rd_d:

begin

c_state <= #1 idle;

cmd_ack <= #1 1'b1;

scl_oen <= #1 1'b0; // set SCL low

sda_oen <= #1 1'b1; // keep SDA tri-stated

sda_chk <= #1 1'b0; // don't check SDA output

end

// write

wr_a:

begin

c_state <= #1 wr_b;

scl_oen <= #1 1'b0; // keep SCL low

sda_oen <= #1 din;  // set SDA

sda_chk <= #1 1'b0; // don't check SDA output (SCL low)

end

wr_b:

begin

c_state <= #1 wr_c;

scl_oen <= #1 1'b1; // set SCL high

sda_oen <= #1 din;  // keep SDA

sda_chk <= #1 1'b0; // don't check SDA output yet

// allow some time for SDA and SCL to settle

end

wr_c:

begin

c_state <= #1 wr_d;

scl_oen <= #1 1'b1; // keep SCL high

sda_oen <= #1 din;

sda_chk <= #1 1'b1; // check SDA output

end

wr_d:

begin

c_state <= #1 idle;

cmd_ack <= #1 1'b1;

scl_oen <= #1 1'b0; // set SCL low

sda_oen <= #1 din;

sda_chk <= #1 1'b0; // don't check SDA output (SCL low)

end

endcase

end

// assign scl and sda output (always gnd)

assign scl_o = 1'b0;

assign sda_o = 1'b0;

endmodule