/////////////////////////////////////////////////////////////////////

////                                                             ////

////  WISHBONE rev.B2 compliant I2C Master byte-controller       ////

////                                                             ////

////                                                             ////

////  Author: Richard Herveille                                  ////

////          richard@asics.ws                                   ////

////          www.asics.ws                                       ////

////                                                             ////

////  Downloaded from: http://www.opencores.org/projects/i2c/    ////

////                                                             ////

/////////////////////////////////////////////////////////////////////

////                                                             ////

//// Copyright (C) 2001 Richard Herveille                        ////

////                    richard@asics.ws                         ////

////                                                             ////

//// This source file may be used and distributed without        ////

//// restriction provided that this copyright statement is not   ////

//// removed from the file and that any derivative work contains ////

//// the original copyright notice and the associated disclaimer.////

////                                                             ////

////     THIS SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY     ////

//// EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED   ////

//// TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS   ////

//// FOR A PARTICULAR PURPOSE. IN NO EVENT SHALL THE AUTHOR      ////

//// OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT,         ////

//// INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES    ////

//// (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF SUBSTITUTE   ////

//// GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR        ////

//// BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF  ////

//// LIABILITY, WHETHER IN  CONTRACT, STRICT LIABILITY, OR TORT  ////

//// (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT  ////

//// OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE         ////

//// POSSIBILITY OF SUCH DAMAGE.                                 ////

////                                                             ////

/////////////////////////////////////////////////////////////////////

//  CVS Log

//

//  $Id: i2c_master_byte_ctrl.v,v 1.8 2009-01-19 20:29:26 rherveille Exp $

//

//  $Date: 2009-01-19 20:29:26 $

//  $Revision: 1.8 $

//  $Author: rherveille $

//  $Locker:  $

//  $State: Exp $

//

// Change History:

//               $Log: not supported by cvs2svn $

//               Revision 1.7  2004/02/18 11:40:46  rherveille

//               Fixed a potential bug in the statemachine. During a 'stop' 2 cmd_ack signals were generated. Possibly canceling a new start command.

//

//               Revision 1.6  2003/08/09 07:01:33  rherveille

//               Fixed a bug in the Arbitration Lost generation caused by delay on the (external) sda line.

//               Fixed a potential bug in the byte controller's host-acknowledge generation.

//

//               Revision 1.5  2002/12/26 15:02:32  rherveille

//               Core is now a Multimaster I2C controller

//

//               Revision 1.4  2002/11/30 22:24:40  rherveille

//               Cleaned up code

//

//               Revision 1.3  2001/11/05 11:59:25  rherveille

//               Fixed wb_ack_o generation bug.

//               Fixed bug in the byte_controller statemachine.

//               Added headers.

//

// synopsys translate_off

`include "timescale.v"

// synopsys translate_on

`include "i2c_master_defines.v"

module i2c_master_byte_ctrl (

clk, rst, nReset, ena, clk_cnt, start, stop, read, write, ack_in, din,

cmd_ack, ack_out, dout, i2c_busy, i2c_al, scl_i, scl_o, scl_oen, sda_i, sda_o, sda_oen, VDD, VSS );

//

// inputs & outputs

//

input clk;     // master clock

input rst;     // synchronous active high reset

input nReset;  // asynchronous active low reset

input ena;     // core enable signal

input VDD;     // Power

input VSS;     // Ground

input [15:0] clk_cnt; // 4x SCL

// control inputs

input       start;

input       stop;

input       read;

input       write;

input       ack_in;

input [7:0] din;

// status outputs

output       cmd_ack;

reg cmd_ack;

output       ack_out;

reg ack_out;

output       i2c_busy;

output       i2c_al;

output [7:0] dout;

// I2C signals

input  scl_i;

output scl_o;

output scl_oen;

input  sda_i;

output sda_o;

output sda_oen;

//

// Variable declarations

//

// statemachine

parameter [4:0] ST_IDLE  = 5'b0_0000;

parameter [4:0] ST_START = 5'b0_0001;

parameter [4:0] ST_READ  = 5'b0_0010;

parameter [4:0] ST_WRITE = 5'b0_0100;

parameter [4:0] ST_ACK   = 5'b0_1000;

parameter [4:0] ST_STOP  = 5'b1_0000;

// signals for bit_controller

reg  [3:0] core_cmd;

reg        core_txd;

wire       core_ack, core_rxd;

// signals for shift register

reg [7:0] sr; //8bit shift register

reg       shift, ld;

// signals for state machine

wire       go;

reg  [2:0] dcnt;

wire       cnt_done;

//

// Module body

//

// hookup bit_controller

i2c_master_bit_ctrl bit_controller (

.clk     ( clk      ),

.rst     ( rst      ),

.nReset  ( nReset   ),

.ena     ( ena      ),

.clk_cnt ( clk_cnt  ),

.cmd     ( core_cmd ),

.cmd_ack ( core_ack ),

.busy    ( i2c_busy ),

.al      ( i2c_al   ),

.din     ( core_txd ),

.dout    ( core_rxd ),

.scl_i   ( scl_i    ),

.scl_o   ( scl_o    ),

.scl_oen ( scl_oen  ),

.sda_i   ( sda_i    ),

.sda_o   ( sda_o    ),

.sda_oen ( sda_oen  ),

.VDD     (VDD),

.VSS     (VSS)

);

// generate go-signal

assign go = (read | write | stop) & ~cmd_ack;

// assign dout output to shift-register

assign dout = sr;

// generate shift register

always @(posedge clk or negedge nReset)

if (!nReset)

sr <= #1 8'h0;

else if (rst)

sr <= #1 8'h0;

else if (ld)

sr <= #1 din;

else if (shift)

sr <= #1 {sr[6:0], core_rxd};

// generate counter

always @(posedge clk or negedge nReset)

if (!nReset)

dcnt <= #1 3'h0;

else if (rst)

dcnt <= #1 3'h0;

else if (ld)

dcnt <= #1 3'h7;

else if (shift)

dcnt <= #1 dcnt - 3'h1;

assign cnt_done = ~(|dcnt);

//

// state machine

//

reg [4:0] c_state;

always @(posedge clk or negedge nReset)

if (!nReset)

begin

core_cmd <= #1 `I2C_CMD_NOP;

core_txd <= #1 1'b0;

shift    <= #1 1'b0;

ld       <= #1 1'b0;

cmd_ack  <= #1 1'b0;

c_state  <= #1 ST_IDLE;

ack_out  <= #1 1'b0;

end

else if (rst | i2c_al)

begin

core_cmd <= #1 `I2C_CMD_NOP;

core_txd <= #1 1'b0;

shift    <= #1 1'b0;

ld       <= #1 1'b0;

cmd_ack  <= #1 1'b0;

c_state  <= #1 ST_IDLE;

ack_out  <= #1 1'b0;

end

else

begin

// initially reset all signals

core_txd <= #1 sr[7];

shift    <= #1 1'b0;

ld       <= #1 1'b0;

cmd_ack  <= #1 1'b0;

case (c_state) // synopsys full_case parallel_case

ST_IDLE:

if (go)

begin

if (start)

begin

c_state  <= #1 ST_START;

core_cmd <= #1 `I2C_CMD_START;

end

else if (read)

begin

c_state  <= #1 ST_READ;

core_cmd <= #1 `I2C_CMD_READ;

end

else if (write)

begin

c_state  <= #1 ST_WRITE;

core_cmd <= #1 `I2C_CMD_WRITE;

end

else // stop

begin

c_state  <= #1 ST_STOP;

core_cmd <= #1 `I2C_CMD_STOP;

end

ld <= #1 1'b1;

end

ST_START:

if (core_ack)

begin

if (read)

begin

c_state  <= #1 ST_READ;

core_cmd <= #1 `I2C_CMD_READ;

end

else

begin

c_state  <= #1 ST_WRITE;

core_cmd <= #1 `I2C_CMD_WRITE;

end

ld <= #1 1'b1;

end

ST_WRITE:

if (core_ack)

if (cnt_done)

begin

c_state  <= #1 ST_ACK;

core_cmd <= #1 `I2C_CMD_READ;

end

else

begin

c_state  <= #1 ST_WRITE;       // stay in same state

core_cmd <= #1 `I2C_CMD_WRITE; // write next bit

shift    <= #1 1'b1;

end

ST_READ:

if (core_ack)

begin

if (cnt_done)

begin

c_state  <= #1 ST_ACK;

core_cmd <= #1 `I2C_CMD_WRITE;

end

else

begin

c_state  <= #1 ST_READ;       // stay in same state

core_cmd <= #1 `I2C_CMD_READ; // read next bit

end

shift    <= #1 1'b1;

core_txd <= #1 ack_in;

end

ST_ACK:

if (core_ack)

begin

if (stop)

begin

c_state  <= #1 ST_STOP;

core_cmd <= #1 `I2C_CMD_STOP;

end

else

begin

c_state  <= #1 ST_IDLE;

core_cmd <= #1 `I2C_CMD_NOP;

// generate command acknowledge signal

cmd_ack  <= #1 1'b1;

end

// assign ack_out output to bit_controller_rxd (contains last received bit)

ack_out <= #1 core_rxd;

core_txd <= #1 1'b1;

end

else

core_txd <= #1 ack_in;

ST_STOP:

if (core_ack)

begin

c_state  <= #1 ST_IDLE;

core_cmd <= #1 `I2C_CMD_NOP;

// generate command acknowledge signal

cmd_ack  <= #1 1'b1;

end

endcase

end

endmodule