/////////////////////////////////////////////////////////////////////

////                                                             ////

////  WISHBONE revB.2 compliant I2C Master controller Top-level  ////

////                                                             ////

////                                                             ////

////  Author: Richard Herveille                                  ////

////          richard@asics.ws                                   ////

////          www.asics.ws                                       ////

////                                                             ////

////  Downloaded from: http://www.opencores.org/projects/i2c/    ////

////                                                             ////

/////////////////////////////////////////////////////////////////////

////                                                             ////

//// Copyright (C) 2001 Richard Herveille                        ////

////                    richard@asics.ws                         ////

////                                                             ////

//// This source file may be used and distributed without        ////

//// restriction provided that this copyright statement is not   ////

//// removed from the file and that any derivative work contains ////

//// the original copyright notice and the associated disclaimer.////

////                                                             ////

////     THIS SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY     ////

//// EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED   ////

//// TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS   ////

//// FOR A PARTICULAR PURPOSE. IN NO EVENT SHALL THE AUTHOR      ////

//// OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT,         ////

//// INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES    ////

//// (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF SUBSTITUTE   ////

//// GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR        ////

//// BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF  ////

//// LIABILITY, WHETHER IN  CONTRACT, STRICT LIABILITY, OR TORT  ////

//// (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT  ////

//// OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE         ////

//// POSSIBILITY OF SUCH DAMAGE.                                 ////

////                                                             ////

/////////////////////////////////////////////////////////////////////

//  CVS Log

//

//  $Id: i2c_master_top.v,v 1.12 2009-01-19 20:29:26 rherveille Exp $

//

//  $Date: 2009-01-19 20:29:26 $

//  $Revision: 1.12 $

//  $Author: rherveille $

//  $Locker:  $

//  $State: Exp $

//

// Change History:

//               Revision 1.11  2005/02/27 09:26:24  rherveille

//               Fixed register overwrite issue.

//               Removed full_case pragma, replaced it by a default statement.

//

//               Revision 1.10  2003/09/01 10:34:38  rherveille

//               Fix a blocking vs. non-blocking error in the wb_dat output mux.

//

//               Revision 1.9  2003/01/09 16:44:45  rherveille

//               Fixed a bug in the Command Register declaration.

//

//               Revision 1.8  2002/12/26 16:05:12  rherveille

//               Small code simplifications

//

//               Revision 1.7  2002/12/26 15:02:32  rherveille

//               Core is now a Multimaster I2C controller

//

//               Revision 1.6  2002/11/30 22:24:40  rherveille

//               Cleaned up code

//

//               Revision 1.5  2001/11/10 10:52:55  rherveille

//               Changed PRER reset value from 0x0000 to 0xffff, conform specs.

//

// synopsys translate_off

`include "timescale.v"

// synopsys translate_on

`include "i2c_master_defines.v"

module i2c_master_top(

wb_clk_i, wb_rst_i, arst_i, wb_adr_i, wb_dat_i, wb_dat_o,

wb_we_i, wb_stb_i, wb_cyc_i, wb_ack_o, wb_inta_o,

scl_pad_i, scl_pad_o, scl_padoen_o, sda_pad_i, sda_pad_o, sda_padoen_o, VDD, VSS );

// parameters

parameter ARST_LVL = 1'b0; // asynchronous reset level

//

// inputs & outputs

//

// wishbone signals

input        wb_clk_i;     // master clock input

input        wb_rst_i;     // synchronous active high reset

input        arst_i;       // asynchronous reset

input  [2:0] wb_adr_i;     // lower address bits

input  [7:0] wb_dat_i;     // databus input

output [7:0] wb_dat_o;     // databus output

input        wb_we_i;      // write enable input

input        wb_stb_i;     // stobe/core select signal

input        wb_cyc_i;     // valid bus cycle input

output       wb_ack_o;     // bus cycle acknowledge output

output       wb_inta_o;    // interrupt request signal output

input	     VDD;	   // Power

input        VSS;	   // Ground

reg [7:0] wb_dat_o;

reg wb_ack_o;

reg wb_inta_o;

// I2C signals

// i2c clock line

input  scl_pad_i;       // SCL-line input

output scl_pad_o;       // SCL-line output (always 1'b0)

output scl_padoen_o;    // SCL-line output enable (active low)

// i2c data line

input  sda_pad_i;       // SDA-line input

output sda_pad_o;       // SDA-line output (always 1'b0)

output sda_padoen_o;    // SDA-line output enable (active low)

//

// variable declarations

//

// registers

reg  [15:0] prer; // clock prescale register

reg  [ 7:0] ctr;  // control register

reg  [ 7:0] txr;  // transmit register

wire [ 7:0] rxr;  // receive register

reg  [ 7:0] cr;   // command register

wire [ 7:0] sr;   // status register

// done signal: command completed, clear command register

wire done;

// core enable signal

wire core_en;

wire ien;

// status register signals

wire irxack;

reg  rxack;       // received aknowledge from slave

reg  tip;         // transfer in progress

reg  irq_flag;    // interrupt pending flag

wire i2c_busy;    // bus busy (start signal detected)

wire i2c_al;      // i2c bus arbitration lost

reg  al;          // status register arbitration lost bit

//

// module body

//

// generate internal reset

wire rst_i = arst_i ^ ARST_LVL;

// generate wishbone signals

wire wb_wacc = wb_we_i & wb_ack_o;

// generate acknowledge output signal

always @(posedge wb_clk_i)

wb_ack_o <= #1 wb_cyc_i & wb_stb_i & ~wb_ack_o; // because timing is always honored

// assign DAT_O

always @(posedge wb_clk_i)

begin

case (wb_adr_i) // synopsys parallel_case

3'b000: wb_dat_o <= #1 prer[ 7:0];

3'b001: wb_dat_o <= #1 prer[15:8];

3'b010: wb_dat_o <= #1 ctr;

3'b011: wb_dat_o <= #1 rxr; // write is transmit register (txr)

3'b100: wb_dat_o <= #1 sr;  // write is command register (cr)

3'b101: wb_dat_o <= #1 txr;

3'b110: wb_dat_o <= #1 cr;

3'b111: wb_dat_o <= #1 0;   // reserved

endcase

end

// generate registers

always @(posedge wb_clk_i or negedge rst_i)

if (!rst_i)

begin

prer <= #1 16'hffff;

ctr  <= #1  8'h0;

txr  <= #1  8'h0;

end

else if (wb_rst_i)

begin

prer <= #1 16'hffff;

ctr  <= #1  8'h0;

txr  <= #1  8'h0;

end

else

if (wb_wacc)

case (wb_adr_i) // synopsys parallel_case

3'b000 : prer [ 7:0] <= #1 wb_dat_i;

3'b001 : prer [15:8] <= #1 wb_dat_i;

3'b010 : ctr         <= #1 wb_dat_i;

3'b011 : txr         <= #1 wb_dat_i;

default: ;

endcase

// generate command register (special case)

always @(posedge wb_clk_i or negedge rst_i)

if (!rst_i)

cr <= #1 8'h0;

else if (wb_rst_i)

cr <= #1 8'h0;

else if (wb_wacc)

begin

if (core_en & (wb_adr_i == 3'b100) )

cr <= #1 wb_dat_i;

end

else

begin

if (done | i2c_al)

cr[7:4] <= #1 4'h0;           // clear command bits when done

// or when aribitration lost

cr[2:1] <= #1 2'b0;             // reserved bits

cr[0]   <= #1 1'b0;             // clear IRQ_ACK bit

end

// decode command register

wire sta  = cr[7];

wire sto  = cr[6];

wire rd   = cr[5];

wire wr   = cr[4];

wire ack  = cr[3];

wire iack = cr[0];

// decode control register

assign core_en = ctr[7];

assign ien = ctr[6];

// hookup byte controller block

i2c_master_byte_ctrl byte_controller (

.clk      ( wb_clk_i     ),

.rst      ( wb_rst_i     ),

.nReset   ( rst_i        ),

.ena      ( core_en      ),

.clk_cnt  ( prer         ),

.start    ( sta          ),

.stop     ( sto          ),

.read     ( rd           ),

.write    ( wr           ),

.ack_in   ( ack          ),

.din      ( txr          ),

.cmd_ack  ( done         ),

.ack_out  ( irxack       ),

.dout     ( rxr          ),

.i2c_busy ( i2c_busy     ),

.i2c_al   ( i2c_al       ),

.scl_i    ( scl_pad_i    ),

.scl_o    ( scl_pad_o    ),

.scl_oen  ( scl_padoen_o ),

.sda_i    ( sda_pad_i    ),

.sda_o    ( sda_pad_o    ),

.sda_oen  ( sda_padoen_o ),

.VDD      (VDD),

.VSS      (VSS)

);

// status register block + interrupt request signal

always @(posedge wb_clk_i or negedge rst_i)

if (!rst_i)

begin

al       <= #1 1'b0;

rxack    <= #1 1'b0;

tip      <= #1 1'b0;

irq_flag <= #1 1'b0;

end

else if (wb_rst_i)

begin

al       <= #1 1'b0;

rxack    <= #1 1'b0;

tip      <= #1 1'b0;

irq_flag <= #1 1'b0;

end

else

begin

al       <= #1 i2c_al | (al & ~sta);

rxack    <= #1 irxack;

tip      <= #1 (rd | wr);

irq_flag <= #1 (done | i2c_al | irq_flag) & ~iack; // interrupt request flag is always generated

end

// generate interrupt request signals

always @(posedge wb_clk_i or negedge rst_i)

if (!rst_i)

wb_inta_o <= #1 1'b0;

else if (wb_rst_i)

wb_inta_o <= #1 1'b0;

else

wb_inta_o <= #1 irq_flag && ien; // interrupt signal is only generated when IEN (interrupt enable bit is set)

// assign status register bits

assign sr[7]   = rxack;

assign sr[6]   = i2c_busy;

assign sr[5]   = al;

assign sr[4:2] = 3'h0; // reserved

assign sr[1]   = tip;

assign sr[0]   = irq_flag;

endmodule